Chandni

- EEG and Photo alignment

  - takes EEG from ch. 3 and 4 *** potentially have an input for channels since they use diff ones

  - aligns and filters based on function

  - align by upsampling photometry then align edges

  - filters the EEG via interpolating the noise

- Photometry Bleaching Preprocessing

  - Does a gaussian filter on both the gcamp and the isosbestic

  - then the gcamp is subtracted by the iso and then divided by the iso data

  - both the gaussian filter and the double exponential fitting will smooth to a degree but the fitting relies on it being an exponential decay

  - The zscore is then taken of the detrended data

- Photometry Motion Preprocessing

  - no motion preprocessing except for an optional filter that does a high butterworth filter

- conclusion

  - the gaussian filter for the bleaching preprocessing is much more elegant and understandable for me if the endgoal is just having a smoother curve

  - no motion preprocessing beside a high filter maybe use the estimation with linear regression in Meiling's

Meiling

- EEG Preprocessing

  - requires user to input file path as well as timepoint for start of seizure and end

  - requires user to select channel for EEG and EMG

  - uses scipy's signal library to do notch filtering for noise cleaning

  - uses filtfilt to do zero phase filtering (forward and backward)

  - notch filtering is for the strong narrow spikes in the time series graph

  - butterworth filtering is then applied using a band from 1Hz to 70Hz

- Photometry Preprocessing

  - gets the file photometry file from folder and filename

  - also has a isosbestic time series to I assume give a stable baseline signal

  - uses butterworth low-pass filtering to smooth the graph *** why only low pass as opposed to band? ***

  - *** will we want to dive into median filtering or is it unnecessary for this data? ***

- photobleaching correction

  - uses a double exponential function to describe the slope of the curve

  - I assume the initial exponent that dominates relates to pre seizure and the following to be during seizure

  - takes the previously filtered data and fits it to a double exponential function using the largest signal as the upper bound

  - does this for both the photometry data as well as the isosbestic

  - the filtered data then has its double exponent fit data subtracted from it to give a version with no trend

  - confirm what is happening here: current assumption is the unnecessary trend is most likely double exponential and by isolating that data and subtracting it we get rid of the trend

  - finally find the linear regression using the isosbestic detrended data to predict the linear data of the detrended GRABne

- motion correction

  - taking the mx+b from the linear regression, apply the isosbestic data as the x values to get an estimation of motion

  - subtract said estimated motion from the detrended data and divide the double exponent fit

- conclusion:

  - Meiling filters, then corrects bleaching and finally corrects motion for preprocessing.

  - filtering used was low pass butterworth

  - bleaching is assumed to exhibit double exponential function behavior

  - motion is estimated based on linear regression

Article

- Filtering

  - Median Filtering

  - Large jumps in the photometry data could be from electrical noise

  - these artifacts can be removed using a median filtering with a window that is 5x that of the spike

  - Zero-phase low pass

  - Used by both researchers in their individual preprocessing *definitely implement in our standard*

  - 2-10Hz typically used for GCamp we should probably just use what the researchers used

- Bleaching Correction

  - Double exponential fitting

  - fitting to a double exponential decay is optimal thus far potentially more complex curves could overfit

  - subtracting from the raw signal is used for when the bleaching is dominated by autofluorescence

  - divide if the bleaching is dominated by the indicator

  - not aware of any systematic characterization difference between the two methods

  - Meiling seems to do both maybe the division is for the normalization and its two birds one stone

  - different isosbestic wavelengths lead to different behavior

    - short wavelengths excite more autofluorescence

    - long wavelengths excite more indicators

    - ***which do we use and is there any special behavior we must address because of such***

- Movement Correction

  - photobleaching should be done before movement and separated from movement

  - use linear regression to predict the signal from the control

- Normalization

  - to normalize we can z-score

  - *** do we use a cpGFP-based indicator ***

  - we can also divide the change in fluorescence signal by the baseline signal level to normalize